ME 605 Computational Fluid Dynamics
Fall 2021-22

Instructor Prof. Dilip Sundaram

Project-1

Name - Devanshu Thakar

Roll no. – 18110174

Contents

Problem Statement	3

Analytical Solution	3

Taylor Series expansion of ϕ	4

Polynomial fitting	5

1. Uniform grid with UDS in convection	5

2. Uniform grid with CDS in convection	7

3. Plot of ϕ vs. x	7

3.1 Plot for  part-1	8

3.1.1 Discussion	8

3.2 Plot for part-2	9

3.2.1 Discussion	9

4. With 41 nodes	10

5 Non-uniform and higher order scheme	11

5.1 Non-uniform grids	11

5.1.1 Using FDS to discretize the diffusion term	12

5.1.2 Using CDS to discretize the diffusion term	13

5.2 Higher Order Schemes	14

5.2.1 Using a fourth order polynomial for diffusive term	14

# Problem Statement

Solution :

Analytical Solution -

ddxρuϕ=ddxΓdϕdx

Given ρ, u and Γ  to be constant, hence independent of x. We have

ρudϕdx=Γd2ϕdx2

Integrating on both sides

ρu ϕx=Γ dϕdx+c1

∴ρuΓ ϕx= dϕdx+c1Γ

The above equation is a linear differential equation, multiplying with Integrating factor e-ρuΓdx=e-ρuxΓ

e-ρuxΓ dϕdx-e-ρuxΓρuΓ ϕx=-c1Γe-ρuxΓ

∴dϕe-ρuxΓdx=-c1Γe-ρuxΓ

Integrating on both sides with x

∴ ϕxe-ρuxΓ=c1ρue-ρuxΓ+c2

ϕxe-ρuxΓ=c1'e-ρuxΓ+c2

ϕx=c1'+c2ePe xL

At x=0, ϕ=ϕo                ϕo=c1'+c2

At x=L ϕ=ϕL                      ϕL=c1'+c2ePe

c2= ϕL-ϕoePe-1   and c1'=ϕO- ϕL-ϕoePe-1

Substituting c2 and c1' and Pe=ρuL/Γ

ϕx=ϕO- ϕL-ϕoePe-1   +  ϕL-ϕoePe-1ePe xL

Therefore we get the exact analytical  solution as -

ϕ-ϕoϕL-ϕo=expPe x/L-1expPe-1

# Taylor Series expansion of ϕ

To obtain the approximations for the first and second order derivatives we use Tayler series expansion

ϕx=ϕxi+x-xi1!∂ϕ∂xi+x-xi22!∂2ϕ∂x2i+x-xi33!∂3ϕ∂x3i+…+x-xinn!∂nϕ∂xni   +H

Substituting x by xi-1 in above equation –

ϕxi-1=ϕxi+xi-1-xi1!∂ϕ∂xi+x-xi22!∂2ϕ∂x2i+H

Neglecting terms higher than 2nd order derivative we get –

∂ϕ∂xi=ϕxi-ϕxi-1xi-xi-1-x-xi2∂2ϕ∂x2i

Assuming uniform grids xi-xi-1=Δx. We get the approximation of the first order derivative by the upwind difference scheme

∂ϕ∂xi=ϕxi-ϕxi-1xi-xi-1+O(Δx)

Approximating Taylor series at both ϕi+1 and ϕi-1 we get –

ϕi=ϕi-1+xi-1-xi1!∂ϕ∂xi+xi-1-xi22!∂2ϕ∂x2i

ϕi=ϕi+1+xi+1-xi1!∂ϕ∂xi+xi+1-xi22!∂2ϕ∂x2i

Combining both the above equations for a uniform grid we get –

∂ϕ∂xi=ϕi+1-ϕi-1xi+1-xi-1

# Polynomial fitting

For deriving a 2nd order approximation of the second order derivative ∂2ϕ∂x2, let ϕ(x) be a polynomial of degree 2.

ϕx=ao+a1x+a2x2

Consider three grid points i-1, i and i+1. Let xi=0 be at the origin. ∴xi-1=-Δx and xi+1=Δx. Assuming x is increasing with i and grid is uniform.

ϕxi=ao       ,  ϕxi-1=ao-a1Δx+a2Δx2   and   ϕxi+1=ao+a1Δx+a2Δx2

∴ϕi=ao           and    Δx2a2-Δx a1=ϕi-1-ϕi           a2Δx2+a1Δx=ϕi+1-ϕi

On solving for a2 and a1 we get

a2=ϕi+1+ϕi-1-2ϕi2Δx2                        and                      a1=ϕi+1-ϕi-12Δx

∴ϕ(x)=ϕi+ϕi+1-ϕi-12Δxx+ϕi+1+ϕi-1-2ϕi2Δx2x2

On differentiating once we get CDS expression

dϕdx=ϕi+1-ϕi-12Δx=ϕi+1-ϕi-1xi+1-xi-1

On differentiating twice we get CDS expression for second derivative

d2ϕdx2=ϕi+1+ϕi-1-2ϕiΔx2

# 1. Uniform grid with UDS in convection

Uniform grid  N=11 nodes, with upwind difference scheme in convective term and CDS in diffusive term

The uniform grid will have equal spacing between the grid points. The grid can be visualized as shown below –

Approximating convective term by upwind (backward)

dϕdxi=ϕi-ϕi-1xi-xi-1

CDS for diffusion term –

d2ϕdx2i=ddxdϕdx=dϕdxi+12- dϕdxi-12xi+1/2-xi-1/2

Approximating the first derivative dϕdxi+12 and dϕdxi-12 by  a CDS scheme –

dϕdxi+12=ϕi+1-ϕixi+1-xi                    and                   dϕdxi-12=ϕi-ϕi-1xi-xi-1

Since the grid is uniform. ∴ Δx=xi-xi-1=xi+1-xi= xi+1/2-xi-1/2

Substituting expression for dϕdxi+12, dϕdxi-12 and Δx in equation – 1

d2ϕdx2i=ϕi+1+ϕi-1-2ϕiΔx2

Substituting the second and first order derivative approximations –

ρuϕi-ϕi-1Δx=Γϕi+1+ϕi-1-2ϕiΔx2

∴Γ ϕi+1+Γ+ρuΔxϕi-1-ρuΔx+2Γϕi=0                  (1)

Equation- (1) is the final form of the discretized equation using upwind (backward) difference scheme for convective term and central difference scheme for the advective term.

Using the notation of AP to denote the principle diagonal, AE (east of AP) upper diagonal of the principle diagonal and AW (west of AP) as the lower diagonal from the principle diagonal.

∴APiϕi+AEiϕi+1+AWiϕi-1=0

Where APi=-ρuΔx+2Γ ;       AEi=Γ      and             AWi=Γ+ρuΔx

At the first interior node (i.e i=2) ϕ1= ϕ(x=0) (boundary condition) is known.

∴ APiϕi+AEiϕi+1=-Γ+ρuΔxϕ1

Similarly at the last interior node –

∴APiϕi+AWiϕi-1=-ΓϕL

In terms on Matrix notation –

A Φ=B

where A= AP1AE1000AW2AP2AE2000.............AE100..AW11AP11  ,    Φ=ϕ1ϕ2:0ϕ11      and          B=-Γ+ρuΔxϕ10:0-ΓϕL

# 2. Uniform grid with CDS in convection

Uniform grid  N=11 nodes, with CDS in convective term and CDS in diffusive term as  well.

The arrangement for the grid points will be the same as in part-1. CDS for the convective term as well. The derivative can be approximated as -  
dϕdxi=ϕi+1-ϕi-1xi+1-xi-1

Since the grid is uniform , xi+1-xi-1=xi+1-xi+xi-xi-1=2Δx

Substituting the approximations for the convective and the diffusive terms

ρuϕi+1-ϕi-12Δx=Γϕi+1+ϕi-1-2ϕiΔx2

∴ρuΔxϕi+1-ϕi-1=2Γϕi+1+ϕi-1-2ϕi

∴-ρuΔx+2Γϕi-1+4Γϕi+ρuΔx-2Γϕi+1=0            2

Equation- (2) is the final form of the discretized equation using central difference scheme for convective term and central difference scheme for the advective term.

∴APiϕi+AEiϕi+1+AWiϕi-1=0

where APi=4Γ ;       AEi=ρuΔx-2Γ      and             AWi=-ρuΔx+2Γ

At the first interior node (i.e i=2) ϕ1= ϕ(x=0) (boundary condition) is known.

∴ APiϕi+AEiϕi+1=ρuΔx+2Γϕ1

Similarly at the last interior node, (ϕ(x=L) =1 ) boundary condition

∴APiϕi+AWiϕi-1=-ρuΔx-2ΓϕL

In terms on Matrix notation –

A Φ=B

where A= AP1AE1000AW2AP2AE2000.............AE100..AW11AP11  , Φ=ϕ1ϕ2:0ϕ11      and       B=ρuΔx+2Γϕ10:0-ρuΔx-2ΓϕL

# 3. Plot of ϕ vs. x

a

b

3.1 Plot for  part-1

Figure 1 Plot of ϕ vs. x computed using 11 uniform nodes (a) with upwind (backward) scheme in convective term, with Pe=50  (b) with upwind (backward) scheme in convective term, with Pe=18

Figure 2 Plot of ϕ vs. x computed using 11 uniform nodes upwind (backward) scheme in convective term. Exact solution for Pe=18 with numerical solution for Pe=50

### 3.1.1 Discussion

From the Figure-1 it can be  observed that for both the values of Pe=50 and 18 the numerically calculated solution is showing high error beyond x=0.6 or 0.7. This is because the ϕ undergoes a sudden change in derivative as x is increased. Also with the increase in Pe it can be seen that ϕ is changing more rapidly for higher x. The Peclet number (Pe) is a ratio of convection (ρuL) and diffusion (Γ diffusivity). Therefore, by reducing Pe we get more diffusion dominated solution. Due to this there is less error for Pe=50 compared to Pe=18.

From Figure-2 it can be seen that the numerical solution for Pe=50 (part-1) is matching very closely to the exact solution with Pe=18. In other words the numerical solution for Pe=50 corrosponds to the exact solution of Pe=18. A lesser value of Pe indicates a diffusion dominated solution. Therefore, it can be said that the upwind (backward) difference scheme (UDS) produces a solution which is having excess diffusion. Thus, the first order UDS is giving over-diffusive solution.

## 3.2 Plot for part-2

The numerical solution by using CDS in diffusive term is shown in Figure-3.

Figure 3 Plot of ϕ vs. x computed using 11 uniform nodes with CDS scheme in convective term (a)  Pe=50 (b) Pe=18

b

a

### 3.2.1 Discussion

It can be observed that for Pe=50 the Central difference scheme in convective term is showing oscillatory nature. By decreasing Pe the oscillatory nature of solution is reduced to a certain extent. However, still the solution is slightly oscillatory as for x=0.7 to x=0.9 the numerical solution is less than analytical solution and after x=0.9 the numerical solution is higher than exact solution. The oscillatory solution in CDS can be attributed to the fact that CDS takes into account the differences from both sides equally. In most of the physical problem there is some sense of direction in which a particular quantality is flowing. CDS is unable to consider the direction of the flow.

Assuming that the flow is from node i-1 to i a backward difference scheme is used. Upwind difference scheme (UDS) is sometimes better than CDS, as it takes into account the direction of the quantity. Moreover, UDS considers the direction in which information is propagating.  The upwind (backward) difference scheme is able to capture a sense of direction of the flow, however, it is giving over diffusive solution.

# 4. With 41 nodes

For uniform grid by increasing the number of grid points, the grid becomes more finer. However, the derivation of the discretized equations remains the same. The only thing that needs to be changed is the value of number of grid points in code Q1.m and Q2.m for part-1 and part-2 respectively. Both the codes were run for 41 uniform grid points with value of Pe=50 and 18. The plots showing ϕ calculated numerically with the analytical ϕ are shown as follows:

Figure 4 Plot of ϕ vs. x using upwind (backward) scheme in convection term and total grid nodes=41 (a) For Pe=50 (b) For Pe=18

a

b

```

Using CDS in convective term –

Figure 5 Plot of ϕ vs. x computed using CDS in convection term and total grid nodes=41 (a) For Pe=50 (b) For Pe=18

a

b

References :

4.1 Discussion

From Figure-4 it can be seen that by increasing the number of node points the error in the upwind difference scheme has reduced. Though the numerical solution is still over-diffusive but the extent of over-diffusiveness is reduced. By making the grid finer more points are taken which reduces the error to some extent. However, the numerical solution still have error in regions where there is sharpen increase in derivative of ϕ (for larger x>0.7). From Figure-5, by refining the grid for CDS in convective term the solution is oscillation free. Therefore refining the grid helps to reduce the error and increases the accuracy of numerical solution.

# 5 Non-uniform and higher order scheme

## 5.1 Non-uniform grids

Truncation error depends not only on the grid spacing but also on the value of the derivative. Therefore, in uniform grid if the value of derivative is high the error would rise. In that situation using a non-uniform grid would be advantageous.

From the plot of ϕ vs. x from previous sections it can be observed that the value of slope remains approximately zero until x=0.7 or 0.8. After that there is a sudden increase in ϕ and its slope. Therefore using coarse grids near x=0 and finer grids near x=1 will help to reduce the error. Let Δxmax be the largest spacing between the two grid points, and r be the expansion factor. In total there are N non-uniform grid points. The summation of all the Δxi s should be equal to L. Let Δx1=Δxmax

Δx1+Δx1r+Δx1r2+ ….+ Δx1rN-1=L=1

Δx1rN-1r-1=1

Since the grid is contracting r<1. Let us choose r=0.7 arbitrarily. Therefore Δx1 can be computed from the above equation. For r=0.7 we get Δx1=0.306

### 5.1.1 Using FDS to discretize the diffusion term

Let the second derivative of the diffusive term be expanded using a FDS (Forward difference scheme). For the inner derivatives,  a BDS (backward difference scheme) is used -

d2ϕdx2i=ddxdϕdx=dϕdxi+1- dϕdxixi+1-xi

d2ϕdx2i=ϕi+1-ϕixi+1-xi- ϕi-ϕi-1xi-xi-1xi+1-xi

d2ϕdx2i=ϕi+1xi-xi-1+ϕi-1xi+1-xi-ϕi(xi+1-xi-1)xi+1-xi2(xi-xi-1)

Approximating the convective term using a BDS

dϕdxi=ϕi-ϕi-1xi-xi-1

Therefore the final discretized equation can be represented as

∴APiϕi+AEiϕi+1+AWiϕi-1=0

APi=ρuxi-xi-1+Γxi+1-xi-1xi+1-xi2(xi-xi-1)                AEi=-Γ xi-xi-1xi+1-xi2(xi-xi-1)

AWi=-ρuxi-xi-1-Γxi+1-xixi+1-xi2(xi-xi-1)

MATLAB code Q5_non_uniform.m was used to solve using non-uniform grid.

Figure 6 Plot of ϕ vs. x computed using 11 non-uniform nodes with upwind (backward) scheme in convective term and FDS in diffusive term, with Pe=50

### 5.1.2 Using CDS to discretize the diffusion term

Applying a Central Difference scheme for the diffusive terms and also for the first order derivatives occurring in it.

d2ϕdx2i=ddxdϕdx≈dϕdxi+12- dϕdxi-1212xi+1-xi-1=ϕi+1-ϕixi+1-xi- ϕi-ϕi-1xi-xi-112xi+1-xi-1

d2ϕdx2i≈ϕi+1(xi-xi-1)+ϕi-1(xi+1-xi)-ϕi(xi+1-xi-1)12(xi+1-xi-1)(xi+1-xi)(xi-xi-1)

Approximating the convective term using a BDS

dϕdxi=ϕi-ϕi-1xi-xi-1

Therefore the final discretized equation can be represented as

∴APiϕi+AEiϕi+1+AWiϕi-1=0

APi=ρuxi-xi-1+Γxi+1-xi-112(xi+1-xi-1)(xi+1-xi)(xi-xi-1)                AEi=-Γ xi-xi-112(xi+1-xi-1)(xi+1-xi)(xi-xi-1)

AWi=-ρuxi-xi-1-Γxi+1-xixi+1-xi2(xi-xi-1)

The same MATLAB program i.e. Q5_non_uniform.m can be used to generate the plot shown below :

Figure 7 Plot of ϕ vs. x computed using 11 non-uniform nodes with upwind (backward) scheme in convective term and CDS in diffusive term, with Pe=50

## 5.2 Higher Order Schemes

## 5.2.1 Using a fourth order polynomial for diffusive term

The second order diffusion term can be discretized by using a higher order central difference scheme. The second order derivative can be obtained by deriving a polynomial of degree four using five points i.e.  ϕi-2, ϕi-1, ϕi, ϕi+1 and ϕi+2 over a uniform grid.  Using a 4th order polynomial to get ϕ

ϕx=ao+a1x+a2x2+a3x3+a4x4

The coefficients ao, a1, a2, a3 and a4 can be obtained by substituting ϕi-2, ϕi-1, ϕi+1 and ϕi+2 into the polynomial expression.

d2ϕdx2i=-ϕi+2+16ϕi+1-30ϕi+16ϕi-1-ϕi-212Δx2

Approximating the convective term using a upwind backward (BDS )

dϕdxi=ϕi-ϕi-1xi-xi-1=ϕi-ϕi-1Δx

Substituting into the 1D convective diffusion equation -

ρuϕi-ϕi-1Δx=Γ-ϕi+2+16ϕi+1-30ϕi+16ϕi-1-ϕi-212Δx2

Γϕi-2-12ρuΔx+16Γϕi-1+12ρuΔx+30Γϕi-16Γϕi+1+Γϕi+2=0

However to use the above expression, the value of ϕ at the second and second last node is required. Therefore using a unwind backward difference scheme to approximate first order derivative and a CDS to approximate second order derivative. From the part-1 we have

∴Γ ϕi+1+Γ+ρuΔxϕi-1-ρuΔx+2Γϕi=0

The values of ϕ2 and ϕn-2 obtained in part-1 are used in this higher order scheme

From the expression we can evaluate ϕ2 and ϕn-1. Note ϕ1=ϕ(x=0) and ϕn=ϕ(x=L) . The final Matrix will look like the following –

where A= -(ρuΔx+2Γ)Γ0000.-(12ρuΔx+16Γ)12ρuΔx+30Γ-16ΓΓ00.0Γ-12ρuΔx+16Γ12ρuΔx+30Γ-16ΓΓ0.00Γ-12ρuΔx+16Γ12ρuΔx+30Γ-16ΓΓ..00.....0:......0:.....00:0..Γ-12ρuΔx+16Γ12ρuΔx+30Γ-16Γ000000(Γ+ρuΔx)-(ρuΔx+2Γ)  , Φ=ϕ2ϕ3ϕ4::ϕn-2ϕn-1      and       B=-Γ+ρuΔxϕ1-Γϕ10:0:0-Γϕn-Γϕn

The MATLAB code Q5_higher_order.m is developed to solve the above matrix equation. The plot of ϕ vs. x for 11 uniform node points for Pe=50  and Pe=18 are shown in Figure--8

Figure 8 Plot of ϕ vs. x computed using 11 uniform nodes with higher order scheme in diffusive term (a)For Pe=50               (b) For Pe-18

a

b

# 5.3 Discussion

# 6. Conclusion